## IOT BASED THREE PHASE POWER MONITORING SYSTEM

## ABSTRACT

The IoT Based Three Phase Power Failure Monitoring System is designed to continuously monitor the condition of a three-phase electrical supply. It uses an ATmega328 microcontroller to detect phase failure, voltage imbalance, under-voltage, and over-voltage conditions. The system displays real-time voltage values on an LCD and sends the data to a cloud platform using a Wi-Fi module. This enables both local and remote monitoring, helping to protect electrical equipment and improve system reliability.

- Monitors three-phase power supply continuously

- Detects phase failure and voltage imbalance

- Uses ATmega328 microcontroller for processing

- Measures R, Y, and B phase voltages

- Displays voltage values on LCD

- Sends data to cloud using Wi-Fi module

- Allows remote monitoring through IoT

- Low-cost, compact, and reliable system

- Prevents damage to electrical equipment

WORKING OF THE SYSTEM

In this system, the R, Y, and B phase voltages are first stepped down using individual transformers and converted from AC to DC using bridge rectifiers. The ATmega328 microcontroller reads these voltages through its analog inputs and compares them with preset threshold values. If any abnormal condition such as phase failure or voltage imbalance occurs, it is immediately detected. The voltage values are shown on an LCD display, and the data is transmitted to the cloud through a Wi-Fi module for remote monitoring. The three-phase supply (R, Y, and B) is connected to individual step-down transformers to reduce high voltage to a safe level.

- The reduced AC voltage from each transformer is converted into DC using a bridge rectifier and filter circuit.

- These DC voltage signals are fed into the analog input pins of the ATmega328 microcontroller.

- The microcontroller continuously measures and compares the voltages with preset threshold values.

- If any abnormal condition such as phase failure, voltage imbalance, under-voltage, or over-voltage is detected, it is identified immediately.

- The real-time voltage values and phase status are displayed on a 16×2 LCD.

- The Wi-Fi module transmits the monitored data to a cloud platform, allowing remote monitoring through a mobile or computer.

- Alerts and historical data help in fault analysis and preventive maintenance.

## APPLICATIONS

This system is widely used in industrial and commercial environments where a three-phase power supply is essential. It is suitable for protecting motors, transformers, and sensitive electrical equipment in factories, substations, hospitals, data centers, and commercial buildings. The system is also useful in smart grid applications and remote power monitoring systems to ensure reliable and safe power operation.

- Industrial motor protection systems

- Power monitoring in factories

- Electrical substations

- Smart grid systems

- Commercial buildings

- Shopping malls

- Hospitals

- Data centers

- Remote power monitoring stations

- Energy management systems